TEST CASES

Simulation and Hardware Verification Plan

Purpose: provide a repeatable checklist for validating the multilevel inverter model and prototype. The expected outcomes below are functional expectations; actual numerical acceptance limits should be filled in after the DC-link voltage, device ratings, and design tolerances are confirmed.

# Test Preconditions

Confirm the circuit schematic and switching table match the implemented hardware/model.

Check DC-source polarity, current limits, wiring, driver supplies, isolation, and emergency stop.

Verify load ratings and use appropriate probes/instruments.

For hardware tests, begin with a low-voltage, current-limited supply and qualified supervision.

# Test Matrix

| ID | Test | Procedure | Expected result / pass criteria | Evidence to save |
| --- | --- | --- | --- | --- |
| TC-01 | Visual and wiring inspection | Inspect power stage, gate drivers, controller, transformer wiring, connectors, and grounding before power-on. | No loose wires, shorts, damaged parts, reversed polarity, or exposed unsafe conductors. | Inspection checklist and photo |
| TC-02 | Controller start-up | Power the controller and driver logic with the power stage DC bus disabled. | Controller starts correctly; gate signals follow the programmed sequence; fault/shutdown state is safe. | Logic analyser or oscilloscope capture |
| TC-03 | Gate-signal interlock | Observe related switch gate signals across transitions. | No unsafe simultaneous conduction; required dead time is present where applicable. | Gate waveform capture |
| TC-04 | DC input verification | Measure each DC source before enabling inverter switching. | All source voltages and polarities match the model/design specification. | Source voltage table |
| TC-05 | Seven-level voltage output | Run simulation or low-voltage hardware with the specified switching pattern and a resistive load. | Output shows the expected sequence of −3Vdc, −2Vdc, −Vdc, 0, +Vdc, +2Vdc, +3Vdc, subject to the topology and source values. | Output voltage waveform |
| TC-06 | R-load current response | Connect the specified resistive load (reported example: 100 Ω) and record current. | Current follows the applied voltage according to the load resistance; no unexplained spikes or instability. | Voltage/current plots and values |
| TC-07 | RL-load response | Use the reported example RL load (R = 100 Ω, L = 50 mH), after confirming ratings. | Current changes more gradually than with the R-only load; output remains stable within design limits. | RL voltage/current plots |
| TC-08 | Positive and negative half-cycle | Observe both polarities of the output waveform. | Positive and negative voltage steps appear in the intended order with the correct polarity. | Full-cycle waveform |
| TC-09 | Zero-level state | Check switching states intended to produce zero output. | Load terminal voltage approaches the designed zero state without unexpected source shorting. | Switch-state log and waveform |
| TC-10 | Load change | Switch between R and RL test cases with power disabled between wiring changes. | Control remains stable and current stays within component/load ratings. | Before/after plots |
| TC-11 | Thermal observation | Run at a conservative operating point for a defined short interval and monitor device temperature. | No abnormal heating, smell, noise, or thermal runaway; stop if limits are approached. | Temperature/time log |
| TC-12 | Protection and shutdown | Test the documented stop/fault response using a safe, controlled procedure. | Gate signals disable safely and the DC source can be isolated promptly. | Shutdown waveform and checklist |
| TC-13 | Simulation-to-prototype comparison | Compare waveform shape and voltage levels at matching DC-source and load settings. | Differences are explained by device drops, dead time, transformer ratio, measurement setup, or model assumptions. | Side-by-side waveform images |
| TC-14 | Repeatability | Repeat a normal operating test at least three times under the same conditions. | Output level sequence and recorded measurements are consistent within the selected tolerance. | Run log |

# Test Record Template

| Field | Record |
| --- | --- |
| Test ID / date |  |
| Model version or hardware revision |  |
| DC source values |  |
| Load type and ratings |  |
| PWM / switching settings |  |
| Instrument and probe details |  |
| Measured voltage/current |  |
| Pass / fail |  |
| Observations and corrective action |  |

# Release Checklist for GitHub

Add the actual Simulink model and controller source files if permitted.

Add prototype photos with clear labels and no personal or confidential information.

Attach output voltage/current waveforms for R and RL loads.

Fill in missing component ratings and actual test measurements.

Do not claim efficiency, THD, or safety compliance unless measured and documented.